VOLUME 4.2

FINANCIAL OFFER TEMPLATES

LUMP SUM CONTRACTS

Introduction

1.	The breakdown of the lump-sum price (Volume 4.2.3) is the itemised list of prices showing the build-up of the price in a lump-sum contract. This breakdown of the lump-sum price does not derogate in any way to the clause stating that, in a lump-sum contract, the total contract price remains fixed irrespective of the quantity of work actually carried out.

The amounts due will be calculated by the tranches specified in article 49(1)(a) of the Special Conditions.

2.	The item description given in the breakdown of the lump-sum price in no way limits the contractor’s obligations under the contract to provide all the works described elsewhere.

3.	The prices of the breakdown of the lump-sum price include all incidental and contingent expenses and all risks necessary to construct, complete and maintain all works in accordance with the contract. Unless separate items are provided in the breakdown of the lump-sum price, prices include all costs involved in the various items of the breakdown.

4.	The lump–sum price and the prices of the breakdown of the lump-sum price are all-inclusive and include any non-exonerated tax or fiscal duty.

VOLUME 4.2.3 — BREAKDOWN OF THE LUMP-SUM PRICE

| Item | Description | Unit | Unit price | Firm quantities | Lump-sum price BGN VAT excl. |
| --- | --- | --- | --- | --- | --- |
| 1 | Construction of a temporary works ground | pcs |  | 1.00 |  |
| 2 | Cutting of shrubs and young forest at thickness of the trees up to 10 cm | 100m2 |  | 6.00 |  |
| 3 | Eradicating single trees dia. 10-15 cm | pcs |  | 17.00 |  |
| 4 | Temporary weaning of construction water | m' |  | 572.00 |  |
| 5 | Construction of temporary approaches to bed | pcs |  | 3.00 |  |
| 6 | Excavation of earth soils at hard condition of an edge (for forming the river bed) for loading and transportation | m3 |  | 840.00 |  |
| 7 | Excavation of earth soils at hard condition of an edge (on the ridge and in the heel of the slopes for placement of the geocells) for loading and transport | m3 |  | 820.00 |  |
| 8 | Excavation, bleeding, loading and transportation of up to 10 km of rock fragments and boulders, at Kraz=1,31 | m3 |  | 685.00 |  |
| 9 | Draining of the trench by water extraction (if necessary) | mc |  | 10.00 |  |
| 10 | Transport of land masses up to 5 km for temporary disposal (Kraz=1.27) | m3 |  | 1066.80 |  |
| 11 | Embankment of earth soils to form the bed of the river | m3 |  | 3217.00 |  |
| 12 | Backfill on the ridge and in the heel of the slopes | m3 |  | 820.00 |  |
| 13 | Compaction of the embankments | m3 |  | 4037.00 |  |
| 14 | Preparation of formwork for bottom thresholds | m2 |  | 110.00 |  |
| 15 | Dismantling formwork for bottom thresholds | m2 |  | 110.00 |  |
| 16 | Supply and installation of concrete C15 / 20 for bottom thresholds | m3 |  | 25.00 |  |
| 17 | Supply and installation of non-woven geotextile | m2 |  | 3015.50 |  |
| 18 | Supply and installation geocell system with hight=10cm | m2 |  | 2900.00 |  |
| 19 | Supply and application of polypropylene ropes (N> 0.9 kN) | m' |  | 2900.00 |  |
| 20 | Supply and application of rigid anchors N14 with length L = 1,0 m | pcs |  | 1200.00 |  |
| 21 | Supply and application of a C16/ 20 concrete fill layer for geocells | m3 |  | 115.00 |  |
| 22 | Layer filling of geocells-earth masses from excavation activities | m3 |  | 100.00 |  |
| 23 | Supply and application of a 5 cm thick clay sealer | m3 |  | 75.00 |  |
| 24 | Laying of hydroseeding | m2 |  | 1850.00 |  |
| 25 | Laying a geocell structure to form a connection with the Struma river and at the bridge start section | pcs |  | 1.00 |  |
|  | TOTAL IN BGN VAT EXCL. |  |  |  |  |
|  | VAT 20%: |  |  |  |  |
|  | TOTAL IN BGN VAT INCL. |  |  |  |  |

Signature:

(a person or persons authorised to sign on behalf of the tenderer)

Date: